Typed Variables in DAVE-ML

input.doc - 21/01/05
Giovanni A. Cignoni

This is a draft proposal for supporting specification of types (int, float, bool, …) in the DAVE-ML variable definitions. In general, the type specification provides more information about the variable. In particular, when the XML source is used to generate code in a language that supports types (C, C++, Java, …) the specification can be exploited to generate more efficient or more accurate code.

In the current DAVEfunc.dtd (v. 1.7b1) the variableDef element is specified as:

<!ELEMENT variableDef 
     (description?, calculation?, isOutput?, uncertainty?)
>
<!ATTLIST variableDef
     name          CDATA	#REQUIRED
     varID         ID	#REQUIRED
     units         CDATA	#REQUIRED
     axisSystem    CDATA	#IMPLIED
     sign          CDATA	#IMPLIED
     alias         CDATA	#IMPLIED
     symbol        CDATA	#IMPLIED
     initialValue  CDATA	#IMPLIED
>

<!ELEMENT variableRef EMPTY>
<!ATTLIST variableRef
     varID         IDREF	#REQUIRED
>

A straightforward solution is to add a new attribute to VariableDef:

…
     type          CDATA	#IMPLIED
     …

In the DTD the type specification is set as optional: old files are still valid. In any case it is the application reading the XML that decides to use or to simply ignore the type info.

The order of the attributes is not important. Probably, to improve human readability, it will be useful to suggest a conventional order, may be varID (as it is important as a reference), name, type, units, initialvalue, … , but this of course is land of personal taste.

It has to be discussed if the value of the type attribute has to be free or constrained to an enumerated set of values. The main problem of an enumerated set of values is backward compatibility: changing the set  in the future may make illegal a number of already existent files. It is important to define a “good” initial set so that future changes, if any, will only add new types.

In the following it is prposed a possible set of values for the type attribute, and, as examples, their interpretation in C/C++ and Java.

| Attribute value | Interpretation | Example C/C++ | Example Java |
| --- | --- | --- | --- |
| int | Default integer | int | int |
| int8 | Signed 8 bit integer | char | byte |
| int16 | Signed 16 bit integer | short | short |
| int32 | Signed 32 bit integer | int or long | int |
| int64 | Signed 64 bit integer | long | long |
| uint8 | Unsigned 8 bit integer | unsigned char | short |
| uint16 | Unsigned 16 bit integer | unsigned short | int |
| uint32 | Unsigned 32 bit integer | unsigned int or long | long |
| uint64 | Unsigned 64 bit integer | unsigned long | long * |
| float | Default floating point | float | float |
| float32 | 32 bit floating point | float | float |
| float64 | 64 bit floating point | double | double |
| float96 | 96 bit floating point | long double * | double * |
| float128 | 128 bit floating point | long double * | double * |
| bigint | Multiple precision integer, such as in the GMP library | mpz_t (GMP) | - |
| bigratio | Multiple precision fraction, such as in the GMP library | mpq_t (GMP) | - |
| bigfloat | Floating point with arbitrary precision mantissa and limited precision exponent, such as in the GMP library | mpf_t (GMP) | - |
| bool | Boolean | bool (std. typedef) | boolean |
| text8 | Text made by ASCII characters | string (std. C++ lib) | string |
| text16 | Text made by characters in an extended coding (e.g. unicode) | - | string |

Notes

Apart of general types with unspecified bit size, size is explicit in the type name. It seems the better way to clearly express the will of the author of the model. Float 96 and 128 are included because there are architectures that implement extended precision in three words (or little less) and other that do in four.

Big numbers, booleans and text types are included in the list for completeness, it has to be decided if they are useful, maybe they can be discarded.

In some cases (maked with a *) the translation in a specific language may result in a loss of data. For instance Java does not have unsigned integers, then an unsigned long cannot always be correctly converted in a long. But this is a problem of the target language.